ཤེས་རིག་དང་རིག་རྩལ་གོང་འཕེལ་ལྷན་ཁག།

ཆུ་མིག་འཕྲུལ་རིག་སྦྱོང་བརྡར་སྤེལ་ཁང་ བུམ་ཐང་།

MINISTRY OF EDUCATION & SKILLs DEVELOPMENT

TECHNICAL TRAINING INSTITUTE-CHUMIG, BUMTHANG

THEORY (BACK PAPER)

Course: Elective Masonry                                                                   Level: National Diploma

Total marks: 100                                                                                              Pass Marks: 50

Invigilator’s Initial

Duration: 3 hrs.

CID No:

DIRECTIONS:

- There are FOUR sections of question and you have to answer ALL.

- Write the answers of section A & B in the question paper itself and write the answers of     section C & D in answer sheet provided.

- Scrabble answers in Section A won’t be counted.

- Any malpractice during the test shall lead to the direct cancellation of the paper with no marks.

For teacher’s use only

| Section | MCQ_A (30) | Other _ B (20) |  |  | SAQ_C (30) | Est._D (20) | Total marks (100) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Marks |  | Q1 (8) | Q2 (6) | Q3 (6) |  |  |  |

SECTION A: Multiple Choice Questions

(30 Marks)

Instruction: Circle the best answer from the following statement provided.

- Which of the following side drains is suitable in an urban area where there is moderate rainfall?

  - L shaped drain

  - U shaped drain

  - V shaped drain

  - Trapezoidal shaped drain

- Which of the following is an example of ergonomic hazards?

  - Poor lighting conditions

  - Chemical exposure

  - Repetitive motions

  - Stress

- Identify the type of brick closure shown in the image.

  - King

  - Queen

  - Mitred

  - Beveled

- What will be lapping zone for slab especially in tension zone?

  - Mid, L/1

  - End, L/2

  - Mid, L/3

  - End, L/4

- The minimum clear cover for a RC column is

  - 25 mm.

  - 40 mm.

  - 50 mm.

  - 75 mm.

SECTION B: Other Types Questions

(20 Marks)

Questions 1                                                                                                          (1*8=8 marks)

c.

b.

a.

d.

e.

f.

g.

h.

Instructions: Identify the parts of staircase and write the answer in the space given.

a. ……………………………………

b. ……………………………………

c. ……………………………………

d. ……………………………………

e. ……………………………………

f. ……………………………………

g. ……………………………………

h. ……………………………………

Question 2                                                                                                           (1*6=6 marks)

Instructions: Write True or False in the black space given.

- The function of a geotextile behind a gabion wall is to prevent soil from washing out through the wall. _______________

- Typical bulk density of cement is 1.44 g/cm3. _______________

Question 3                                                                                                           (1*6=6 marks)

Instructions: Fill in the blank with appropriate word(s)

- The positive bending moment usually formed in beam due to stress is termed as __________________________________

SECTION C: Short Answer Questions

(30 marks)

- What is workability of fresh concrete?                                                                     (1 mark)

- How does implementation of 5S help in daily chores? Explain a few examples.  (2 marks)

- Write a difference between one-way slab and two-way slab in terms of the following:

(2 marks)

- Span ratio

- Load transfer

SECTION D: Estimation Type Questions

(20 marks)

- Calculate the weight of main bars in column 6#16Ø of 36 m length?                   (2 marks)

- Calculate the listed materials given below if the wet volume of M15 concrete grade is 50 ft3.                                                                                                                            (7 marks)